Multi-plane co-registration

---------------------------

Summary of method

-----------------

The 2 image data sets, D1 & D2, to be co-registered are first put in the

same physical unit.  Then by keeping D2 fixed in space while allowing

adjustments in orientations for window planes of D1, D1 is compared and

matched up with D2.  The window images of D1 are superimposed with their

corresponding co-window images and they can be easily compared since their

pixels have the same co-registered coordinates. By dragging the computer mouse,

the superimposed images are matched up with the original images repectively.

And each superimposed match from the windows suggests a relative move of D1,

and D1 is then moved relatively and that in turn causes changes in all

window planes of D1. Since the windows of D1 are locked so that they always

have same distances and angles among them, and so is true with their

corresponding co-windows, the new superimposed matches therefore always

suggest how D1 should be moved next.  In addition, by flipping the planes

slightly from any locked windows of D1, the corresponding changes in

orientation in the other locked windows of D1 are automatically made.  And their

new superimposed images are then dynamically computed to further suggest

how well D1 & D2 match.  After the matching is completed, the result

of the co-registration can be verified through the different view cuts

and the corresponding co-windows of both D1 & D2 should always show similar

image shapes.

Steps

------

1) Open up any 2 image data sets to be co-registered in 3D, name them D1 & D2.

2) Resize the one with the bigger voxel size to the smaller one...

a) so that they can be compared on the same physical units.

b) resizing to the smaller one so that you can have a better resultion.

3) Matching up D1 & D2 by aligning them on a common 3D image center

and rotation axis....

a) 5 windows show D1 images & 3 show D2 images. Of course, any number

of windows used is arbitrary.  Let all windows related to D1 be

labelled with prefix "W", and "V" for windows related to D2.

By the reslicing options, we now make 1 window each from D1 & D2

(call them W1 & V1) show roughly the same view, e.x., both sagittals,

or both axials and so on.  They do not need to be on the

exact orientation, both on approximately the same view so that a

better image perception can be obtained when using them  to aligne

the 3D image centers of both D1 & D2 by using the image superimposing

options described below.

b) Superimpose V1 on W1.  From W1, drag the superimposed V1 to a

position where the 3D image centers of both roughly aligned.  When

the centers align, the superimposed W1 & V1 should show that

their image shapes at least roughly match.

c) At this point, based on the superimpoed move just made, a relative

move in D1 can be made so that W1 can align with V1 the same way

even without the superimposed move.  The user then click a button

to move D1 relatively.  This action makes the relative changes

in all windows related to D1, based on their plane position &

orientation.

d) Co-register W1 with V1 so that any change made in the plane orientation

of W1 will cause the corresponding change in V1 and they always

have the same co-registered coordinate positions for their window

pixels.

e) Now from W2 of D1, make the angle & distance of W1 & W2 to be the

same as V1 & V2.  After this step W2 is locked to W1, & V2 locked to

V1, namely any change made in the orientation of W1 will cause a

relative change in W2 with the same angle & distance between them

maintained, and the same applies for V1 & V2 since they are locked.

And at this point, W2 is co-registered with V2.  Do the same step for

W3 & V3, just like for W2 & V2.  Now W1 is the co-window for V1,

W2 for V2, W3 for V3, and so is the reverse.

f) Deactivate the actiaon of co-registration between all the co-windows,

so that D2 can be fixed in space while D1 is being adjusted so that

their centers align.

g) Superimpose V2 on W2, and V3 on W3.  And repeat steps b) & c)

from W1, W2 or W3 until the image centers of D1 & D2 have finally

aligned, where all the superimposed images are finally matched

with their corresponding shapes.  If the superimposed shapes

differ too much, flip and change the orientation or plane

position of either W1, W2 or W3 slightly.  Remember that since they

are locked, a change made in one causes to the relative change

in the others.  And again, from the new superimposed shapes

generated after the change, the decision of matching up D1 & D2

are actually made based on the image shapes and features from

different orientations simultaneously.

4) Checking the result of co-registration of D1 & D2 after they aligned on

a common image center and rotation axis, from the different image view cuts...

a) Activate the co-registration again between all the co-windows.  New

windows of defferent view and their corresponding co-windows can also

be added now like in step e) of 3), and no further alignment of

the common image center and rotation axis is needed.

b) From W4 or W5 which is not locked to any window, change the orientation

of either W1, W2 or W3 and check the resulting images & superimposed

shapes in all of co-windows since they are locked and co-registered.

All corresponding co-windows should now show the same image shapes

if D1 is correctly co-registered to D2.  Some easy-to-check cuts like

those showing the eye balls, holes in the images can be made for

convincing verifications of the result.